Concept Note: Multi-Objective Sizing for Hybrid Solar Systems Using Synthetic Residential Loads and Machine-Learning-Based Long-Term Solar Irradiance Forecasts

1. Overview of the Study Area

Study Context

This study focuses on hybrid solar systems for residential households, where rooftop PV, a battery storage unit, and the utility grid are coupled via a smart inverter to supply domestic electrical demand [1].

Geographic Focus

The work is situated in Kenya, using a representative hybrid solar supplied household as a case study to capture typical appliance ownership, usage patterns, and electricity consumption levels in urban and peri-urban settings.

Target Users

The framework is intended for use by solar installers and system designers who size hybrid systems for Kenyan households and need a practical, data-driven way to move from a simple appliance list and site location to concrete PV, battery, and inverter sizes.

2. Problem Definition

Existing Challenges

Challenge 1: Lack of Electrical Load Profiles
Installers currently rely on appliance lists and rough estimates rather than time-resolved load profiles to size systems [2].

In most residential sites, power meters are absent, leaving household electricity demand unmeasured and unknowable [3].

Load monitoring campaigns, the conventional approach to gather demand data, are impractical because clients are unwilling to delay system installation for extended monitoring periods (typically weeks to months).

Challenge 2: Solar Irradiance Variability and Single-Value Design Estimates
System designers typically rely on a single average sun-hours value when estimating PV production, which masks the substantial temporal variability in solar irradiance [4]. This oversimplification leads to inaccurate estimates of future PV energy yield.

Without better characterization of irradiance variability and improved long-term forecasting, designers cannot reliably predict system performance across the intended system lifespan [5], [6].

Challenge 3: Installation Cost, Autonomy, and Grid Dependence Trade-offs
Installers must navigate trade-offs between keeping upfront capital cost low and maximizing the share of household energy supplied by PV and battery instead of the grid (which simultaneously drives bill savings and perceived autonomy under Kenya’s largely flat residential tariffs).

Existing sizing methods mostly optimize lifecycle economic metrics rather than treating upfront cost and annual grid energy offset as the primary decision axes that matter to Kenyan households and installers [7].

Existing tools and papers typically present results as full Pareto fronts or technical performance metrics, rather than summarizing them into a few intuitive options (e.g., “cheapest”, “balanced”, “high-autonomy”) that match how installers actually communicate choices to clients [7].

Knowledge Gaps

- Lack of an integrated framework that combines synthetic load modeling, long-term solar forecasting, and multi-objective sizing

- Limited methods adapted to real installer constraints (no meters, no detailed irradiance records)

- Few tools translate technical sizing outcomes into simple options showing cost–autonomy–grid dependencies

- Minimal incorporation of forecast uncertainty into risk-aware sizing decisions

3. Objectives of the Study

Main Objective

To develop an integrated framework that combines residential load modelling, long-term solar irradiance forecasting, and multi-objective optimization for sizing hybrid solar systems.

Specific Objectives

1: To develop and validate a robust model for generating realistic synthetic residential electricity load profiles.

2: To develop and validate a machine learning model for long term solar irradiance forecasting.

3: To develop and validate a multi objective optimization method for sizing hybrid solar systems.

4. Methodology

4.1 Overall Framework Structure

The framework operates on a clear input-process-output model:

Inputs:

- Appliance list and wattages (from household survey)

- Time-of-use probabilities (from occupancy and usage data)

- Site location (for solar resource characterization)

Processes:

- Synthetic load profiling

- Machine learning long-term solar forecasting

- Multi-objective sizing optimization

Outputs:

- PV array size, battery capacity, and inverter rating

- Three practical design options: cheapest, balanced, high-autonomy

- Key performance indicators for each option

4.2 Phase 1: Residential Load Modelling

| Aspect | CREST 1.0B | MECS-CREST | Proposed Model |
| --- | --- | --- | --- |
| Resolution | 1-minute | 1-minute | 1-minute |
| Household Rep. | Random synthetic household details per run | Fixed baseline household details per user story | Fixed baseline household details per household |
| No. of Residents | User defined (1-6) | User defined (cooking diary based) | Survey-specified fixed number |
| Day | Weekday & Weekend | Single day | Weekday & weekend day × MC diversity x seasonal scaling |
| Clearness | Loughborough clearness stats for lighting | PVGIS clearness stats for lighting | NASA POWER clearness stats for lighting |
| Data Sources | UK TUS/ownership stats for average household | Kenyan Cooking diaries + simplified TUS for average household | Specific household TUS/ownership data from surveys tailored for household accuracy |
| Development Flow | Calibrated to national aggregates | Cooking diaries drive parameters | Household surveys drive parameters |
| Occupancy | Markov transitions from (UK tables) | Fixed occupancy profile built from cooking diaries. | Markov transitions from (Household data) |
| Appliance Handling | Random ownership/power distributions | User-specified + cooking appliances | Survey-specified ownership |
| Appliance model | TUS activity probabilities | Simplified TOU + cooking | Survey TUS activity probabilities |
| Lighting model | Irradiance threshold + random bulbs | Specified LED configurations + irradiance | Survey bulbs + NASA irradiance |
| Validation Data | Aggregate UK statistics | Illustrative examples only | Independent case-study measurements |
| Diversity/Scale | Aggregate many runs | Change parameters within/ across households | Monte Carlo ensembles |
| Implementation | Excel/VBA spreadsheet | Modified Excel | Python |
| Validation Metrics | Calibrated to UK aggregates | Visual examples (Kenya diaries) | MAE, var, P/A ratio, vs measured case-study |
| Output | Single 24h load profile | 24h household + aggregate profiles | Weekday + weekend statistical load profiles |

Step 1: Household Survey and Data Collection

- Select a representative case-study household

- Conduct structured survey to collect: appliance list with rated power, typical usage patterns, occupancy schedules, dwelling characteristics (size, orientation, thermal properties)

Step 2: Field Load Monitoring

- Install a single-phase DIN-rail power meter to continuously measure household load profile

- Record load data at 1 hour resolution

- Capture at least 1 month of continuous monitoring to capture weekday/weekend variations.

Step 3: Data Cleaning and Preprocessing

- Remove or interpolate measurement gaps and outliers.

- Standardize timestamps and data formats to an hourly time series consistent with the synthetic load model.

- Implement data cleaning and preprocessing in Python using libraries such as pandas and NumPy, and store processed datasets in a structured format (e.g., CSV) for subsequent modelling and optimization.

Step 4: Stochastic Bottom-Up Load Model (CREST-Inspired)

- Develop Python-based stochastic bottom-up model using CREST framework: Markov occupancy simulation (time-of-day transitions: "nobody home", "one person", "multiple active") [8].

- Configure the model to operate at an hourly time step and run many Monte Carlo realizations to produce ensembles of synthetic demand profiles over the design horizon, capturing variability in daily and weekly residential electricity use.

- Calibrate the Python implementation against the original open-source Excel/VBA tool from the Centre for Renewable Energy Systems Technology (CREST) by running both on a common reference household and matching key statistics (annual energy, peak demand, daily profiles, load duration curves) [10].

Step 5: Synthetic Profile Generation and Validation

- Generate hourly synthetic electricity demand profiles for the case-study household across the target system design horizon (e.g., 1 year)

- Validate synthetic profiles against the measured reference load using metrics: mean absolute error, peak demand accuracy, daily and weekly variance, peak-to-average ratio

- Visual comparison of synthetic vs. measured load duration curves and daily patterns

4.3 Phase 2: Long-Term Solar Irradiance Forecasting

Step 1: Data Acquisition

- Access long-term historical Global Horizontal Irradiance (GHI) and temperature data from NASA Power Dataset for the study location

- Time span: minimum 20 years of monthly or daily data to capture long-term variability and trends

Step 2: Data Preprocessing

- Gap filling for missing data points using appropriate interpolation or statistical methods

- Outlier detection and handling

- Feature engineering: compute derived variables (e.g., clearness index, temperature anomalies)

- Divide data into training, validation, and test sets (e.g., 60/20/20 split)

Step 3: Machine Learning Model Development

- Train multiple candidate models for long-term irradiance forecasting:

  - Convolutional Neural Networks (CNN) [11]

  - Long Short-Term Memory (LSTM) [11].

  - Random Forest: baseline non-deep-learning method [12]

  - Support Vector Regression (SVR): alternative regression approach [12]

- Models should predict monthly or yearly average GHI for 10+ year forecasting horizon

Step 4: Model Evaluation and Selection

- Evaluate all trained models on the held-out test set using metrics:

  - Root Mean Square Error (RMSE)

  - Correlation coefficient (R²)

  - Mean Absolute Error (MAE)

- Select the best-performing model based on test set accuracy

- Benchmark against baseline methods (persistence forecast, historical mean)

Step 5: Uncertainty Quantification and Forecast Generation

- Quantify forecast uncertainty through ensemble methods or prediction intervals

- Generate long-term solar irradiance forecasts for the system design horizon

- Convert irradiance forecasts to PV energy yield time series using standard PV performance equations

4.4 Phase 3: Multi-Objective Sizing Optimization

Step 1: System Model Formulation

- Develop a hybrid PV–battery–grid simulation model with the following components:

  - PV array: converts synthetic solar irradiance forecasts into electrical energy

  - Battery storage: smooths load-generation mismatches, decision variables include capacity (kWh) and power rating (kW). The study assumes lithium iron phosphate (LiFePO₄) batteries, reflecting their high cycle life and safety [13].

  - Grid connection: supplies unmet demand or absorbs excess PV generation. (240 V AC, 50 Hz)

  - Inverter: converts DC to AC and manages power flow, decision variable is inverter rating (kW)

Step 2: Decision Variables
Define three continuous decision variables:

- PPV: PV array size (kW)

- EBatt: Battery energy capacity (kWh)

- PInv: Inverter power rating (kW)

Step 3: Objective Functions
Formulate two competing objectives:

- Minimize upfront system cost: Minimize total initial capital cost of PV, battery, inverter, BOS, and installation (KES).

- Maximize annual grid energy offset / self-sufficiency: Maximize the annual household load served by PV + battery instead of the grid (kWh/year), which in Kenya:

- is directly proportional to annual bill savings under flat residential tariffs, and

- is also a good proxy for autonomy/self-sufficiency

Step 4: Multi-Objective Optimization Algorithm

- Apply Non-dominated Sorting Genetic Algorithm II (NSGA-II) to generate Pareto front of non-dominated designs —solutions where no objective improves without worsening another [14].

Step 5: Pareto Analysis

- Extract 3 options: cheapest (min cost), balanced (Pareto centre), high-autonomy (max grid independence).

- Report KPIs per option: cost (KES), grid import (kWh/yr), self-sufficiency ratio, sizes (PV, LiFePO₄ battery, inverter).

4.6 Framework Validation

- Use case-study household to generate three designs (cheapest, balanced, high-autonomy).

- Simulate each over representative year; compare grid import, self-sufficiency, bill savings vs. measured data.

- Sensitivity analysis on PV/LiFePO₄ costs and tariffs; state assumptions/limitations.

5. Expected Outcomes

- Validated synthetic load model for realistic household profiles.

- Accurate ML-based long-term GHI forecasts with uncertainty.

- Three Pareto sizing options (cheapest/balanced/high-autonomy) with KPIs.

- Installer decision-support tool prototype

References

[1] F. Fiedler, “Techno-economic analysis of grid-connected PV–battery systems for single-family homes,” M.Sc. thesis, Chalmers Univ. of Technology, 2022.

[2] S. F. Ahmed, M. E. Elnaggar, and H. A. Ziedan, “Novel and cost-efficient design of stand-alone PV system using PVsyst,” Scientific Reports, vol. 15, no. 1, article 12345, 2025.

[3] N. J. Williams, “Microgrid utilities for rural electrification in East Africa,” PhD thesis, Carnegie Mellon Univ., 2017.

[4] M. H. Ghodusinejad, H. Yousefi, and M. H. Jahangir, “A systematic review of solar irradiance forecasting across time horizons using physical, satellite, and AI-based methods,” Solar Communications, 2025.​
[5] R. Mabodi, A. Bagheri, and M. T. Masouleh, “Solar irradiance forecasting for informed solar systems planning using machine learning models,” South African Journal of Science, vol. 120, no. 3–4, 2024.
[6] S. M. H. Kazemi, M. H. Savoji, and A. A. Safavi, “Solar radiation prediction: A multi-model machine learning and deep learning approach,” AIP Advances, vol. 15, no. 5, 055201, 2025.

[7] B. Pirouz, F. Guerriero, and A. Algieri, “Optimal sizing and operation of battery energy storage systems in renewable energy systems using multi-objective optimization and artificial intelligence,” SSRN Working Paper, Jul. 2025.

[8] H. Page, “Four-state domestic building occupancy model for energy demand simulations,” Building Simulation, 2021.

[9] M. Leach, C. Mullen, and N. Wade, “Household electricity load modelling: Cooking and non-cooking,” MECS Working Paper, 2020.

[10] A. D. Wills, I. Beausoleil-Morrison, and V. I. Ugursal, “Adaptation and validation of an existing bottom-up model for simulating temporal and inter-dwelling variations of residential appliance and lighting demands,” Journal of Building Performance Simulation, vol. 11, no. 3, pp. 343–358, 2018.

[11] N. Azizi, M. Yaghoubirad, M. Farajollahi, and A. Ahmadi, “Deep learning based long-term global solar irradiance and temperature forecasting using time series with multi-step multivariate output,” Renewable Energy, vol. 206, pp. 135–147, 2023.

[12] R. Sharma, P. Chaudhary, and A. Kumar, “Prediction of solar power generation using support vector regression and random forest regression,” International Research Journal of Modernization in Engineering, Technology and Science (IRJMETS), vol. 6, no. 5, pp. 1234–1241, 2024

[13] T. Zhao, Y. Wang, and J. Li, “An overview on the life cycle of lithium iron phosphate,” Chemical Engineering Journal Advances, vol. 18, 100562, 2024.

[14] R. Ramakrishnan, S. Devaki, K. R. Ranjith, and P. Suresh, “Multi-objective optimization-based PV-battery storage system sizing with a judicious excess energy transfer framework fostering net-zero energy buildings,” Electric Power Systems Research, vol. 238, 110398, 2025